DICOM Correction Proposal

| STATUS | Letter Ballot |
| --- | --- |
| Date of Last Update | 2024/08/24 |
| Person Assigned | steven.nichols@gehealthcare.com |
| Submitter Name | steven.nichols@gehealthcare.com on behalf of WG-14 |
| Submission Date | 2024/03/06 |

Correction Number CP-2399

Log Summary: Correct Audit Message Schema Conflict

Name of Standard PS3.15

Rationale for Correction: The optionality of the ParticipantObjectName and ParticipantObjectQuery elements in several DICOM messages conflicts with the requirements specified in section A.5.1.1 of the Audit Message Schema. The schema requires either a name or a query ID field, however, in the message specifications, they are both user optional (U). Optionality for these elements in Table A.5.2-1. General Message Format, for example, is U. This CP corrects the optionality of ParticipantObjectName and ParticipantObjectQuery.

Correction Wording:

Note to reviewers: the schema specification for ParticipantObjectName and ParticipantObjectQuery is included for reference.

### A.5.1.1 Audit Message Schema

The following is the content of the audit schema:

…

ParticipantObjectIdentificationContents =

element ParticipantObjectIDTypeCode { CodedValueType },

(element ParticipantObjectName { token } |             # either a name or

element ParticipantObjectQuery { xsd:base64Binary }),  # a query ID field,

…

Modify Table A.5.2-1. General Message Format as follows:

Table A.5.2-1. General Message Format

|  | Field Name | Opt. | Description | Additional Conditions on Field Format/Value |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| Participant Object (multi-valued) | ParticipantObjectName | U MC | An instance-specific descriptor of the Participant Object ID audited, such as a person's name. | Usage refined by individual message descriptions Required if ParticipantObjectQuery is not present. |
|  | ParticipantObjectQuery | U MC | The actual query for a query-type participant object. | Usage refined by individual message descriptions Required if ParticipantObjectName is not present. |

Modify Table A.5.3.2-1. Audit Log Used Message as follows:

Table A.5.3.2-1. Audit Log Used Message

|  | Field Name | Opt. | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| Participating Object: Identity of the audit log (1) | ParticipantObjectName | U M | Shall be: "Security Audit Log" |
|  | ParticipantObjectQuery | U | not specialized |
| … |  |  |  |

Modify

Table A.5.3.3-1. Audit Message for Begin Transferring DICOM Instances,

Table A.5.3.4-1. Audit Message for Data Export

Table A.5.3.5-1. Audit Message for Data Import

Table A.5.3.6-1. Audit Message for DICOM Instances Accessed

Table A.5.3.7-1. Audit Message for DICOM Instances Transferred

Table A.5.3.8-1. Audit Message for DICOM Study Deleted

Table A.5.3.15-1. Audit Message for Procedure Record

as follows:

Note: Each message, the first Participating Object references the Study. The second Participating Object references the Patient.

|  | Field Name | Opt. | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| … | ParticipantObjectName | U MC | not specialized Required if ParticipantObjectQuery is not present. |
|  | ParticipantObjectQuery | U MC | not specialized Required if ParticipantObjectName is not present. |
| … |  |  |  |
|  | ParticipantObjectName | U M | The patient name |
|  | ParticipantObjectQuery | U | not specialized |

Modify Table A.5.3.10-1. Audit Message for Query as follows:

Table A.5.3.10-1. Audit Message for Query

|  | Field Name | Opt. | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| Participating Object: SOP Queried and the Query (1) | ParticipantObjectName | U | not specialized |
|  | ParticipantObjectQuery | M | If the ParticipantObjectIDTypeCode is (110181, DCM, "SOP Class UID"), then this field shall hold the Dataset of the DICOM query, xs:base64Binary encoded. Otherwise, it shall be the query in the format of the protocol used. |
| … |  |  |  |

Modify Table A.5.3.11-1. Audit Message for Security Alert as follows:

Table A.5.3.11-1. Audit Message for Security Alert

|  | Field Name | Opt. | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| Participating Object: Alert Subject (0..N) | ParticipantObjectName | U M | not specialized |
|  | ParticipantObjectQuery | U | not specialized |
| … |  |  |  |

Modify

Table A.5.3.13-1. Audit Message for Order Record

Table A.5.3.14-1. Audit Message for Patient Record

as follows:

|  | Field Name | Opt. | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| Patient (1) | ParticipantObjectName | U M | The patient name |
|  | ParticipantObjectQuery | U | not specialized |
| … |  |  |  |